- Open the cover of PROFI-8455

- Close the JP4 and pin 1, 2 (JP5) by jumper

- Connect the power supply

- Connect the PC and the PROFI-8455 by Com port(RS-232) and turn the power on

- Change Comport setting in the 7188xw.ini 	EX:com6 you should set C6

- Run 7188xw.exe and key in “enter”. You must confirm the PROFI-8455 which is in Command Mode.

- Press “F4” and start update firmware

- Update success, and turn the power off

- Open JP4 and pin 2, 3 (JP5) by jumper

- Close the cover of PROFI-8455