- Open the cover of I-7550-E

- Close the flash protection pin 1, 2 (JP2) by jumper

- Connect the power supply

- Connect the PC and the I-7550-E by Ethernet(RJ-45) and turn the power on

- Run MiniOS7 Utility and Press F12 to find IP address of the I-7550-E.

- Stop scan and close scan page. Press “Connection” button.

- Select “TCP” and renter the IP address of the I-7550-E, then press “OK” button.

- Right click on the right side window, and select “Erase Disk”. Press “Yes” when the confirm dialog pops.

- Open File explorer and select 7 files: 7550Evxx.EXE, autoexec.bat, bottom.htm, bottom.jpg, main.htm, top.htm, and top.jpg. Drag these 7 file to the right-side window of MiniOS7 Utility.

- Update success, and turn the power off

- Close the flash protection pin 1, 2 (JP2) by jumper

- Close the cover of I-7550-E