1. Download EXE file from PC into 7188's RAM-DISK

2. Use COM4 & inturrupt-driven driver to download

--> 7188's COM4 input  = PC's keyboard

--> 7188's COM4 output = PC's monitor

3. This method is high speed(faster than loadram2.exe)

4. Working steps are give as following:

1.  Power-off PC & 7100

2.  Install the download cable between 7188's COM4 & PC's COM1

3.  Power-on PC

4.  Cd \7188 (in PC)

5.  Run 7188.exe (in PC)

6.  Power-on 7188

7.  You will see the 7188's power-on mesage ares shown on PC's monitor

8.  Keyin "ld3" & Enter-key in PC's keyboard

9.  Keyin "demo1.exe" & Enter-key in PC's keyboard

10. Keyin ALT-S in PC's keyboard

11. Keyin "demo1.exe" & Enter-key in PC's keyboard

12. Keyin "b:" & Enter-key in PC's monitor

13. Keyin "dir" & Enter-key in PC's monitor

14. Keyin "demo1.exe" & Enter-key in PC's monitor

Now you execute demo1.exe in RAM-DISK.